Section 18 PACTOR OPERATION

PACTOR is a new operating mode added with version 5.5 of the

PC-PAKRATT II program.  To operate PACTOR you must have a

PK-232 MBX, PK-900, DSP-1232 or DSP-2232 data controller with the

optional PACTOR firmware.  If the firmware in your data controller

does not support PACTOR, you will not be able to use this operating

mode.

PACTOR combines many of the best features of Packet and AMTOR

together into a new mode which is well suited to the difficult and

noisy conditions found on the HF bands.  Like packet, PACTOR uses

the ASCII character set and allows 7-bit files to be transmitted.

It is helpful to think of PACTOR like an extended version of AMTOR

that includes both upper and lower case characters.  Like AMTOR,

PACTOR is most reliable when two stations are connected in an ARQ

handshaking fashion.  If you are familiar with AMTOR operation,

PACTOR will be a snap.

Be sure you have read and understand the information in your data

controller manual for PACTOR operation.  Also be sure you have

properly installed the PC-PAKRATT II program as described in the

Installation section of the PC-PAKRATT II Operating Manual.

To start the PC-PAKRATT II program do the following:

1.  Turn your computer ON.

2.  Move to the PCPAKRAT directory on your computer.

(At the DOS prompt, type CD\PCPAKRAT and press <ENTER>.)

3.  At the new DOS prompt, type PP and press <ENTER>.

PC-PAKRATT II will load and you should see the Communications Menu.

If you see any error messages during the initialization process,

just press any key to finish the initialization.  If you have

properly configured the system as described in Section 4 (SYSTEM

STARTUP) of the PC-PAKRATT II operating manual, you should see no

errors.

THE PACTOR SCREEN

At the Communications Menu, select the "PACTOR Screen" option.

The PACTOR screen similar to the one shown below will be displayed.

18-1

����������������������������������������������������������������ͻ

�                                                                �

�                                                                �

�                                                                �

�                                                                �

�                                                                �

�                                                                �

�______________________ESC to Exit______________________000 %____�

�                                                                �

�                                                                �

�����ALT-H:Help���ALT-M:Macro Keys���CTRL-Pg-Up:Clear Buffer����Ͷ

� PACTOR      100                        Mdm:OFF TMail:OFF xx:xx �

� Recv                     PACTOR  Standby   PT200:ON      xx:xxZ�

�___ALT-L:MiniLog____PgUp:Scroll Back____Home:Log____xx/xx/xx____�

����������������������������������������������������������������ͼ

Like the Packet screen, the PACTOR screen is divided into  three

sections.  The top area is the Receive window and its size is

determined by what you specify in the PC-PAKRATT Parameters menu.

This is where you will see the information being transmitted to you.

The middle section, just below the "ESC to Exit" line is the Send window.

What you type into this area is what will be transmitted.  Its size is

what's left of the available space after defining the receive window.

The bottom portion is the Status window.  It contains the current

PACTOR status indicators.  These are described below.  Remember

you can use the mouse to click on any of the items discussed below.

The <ALT><H> key combination takes you to the PACTOR Help screen.

When you are viewing the HELP screen, you can use the mouse to

point to any of the listed items and click once with the left

mouse button to "do" whatever that item does (e.g., click on

"Tmail:OFF" to enable or disable the PACTOR MailDrop).  You

can also simply type the highlighted letter to enable the

function.  When you click on <ESC>, however, you will only go

back to the PACTOR screen, not exit to the Communications

Menu.  If you click on or press <ESC> while the PACTOR screen

is displayed, then you will go to the Communications Menu.

Refer to the "PACTOR Help Screen" paragraphs later in this section

for more information.

PACTOR indicates that you are operating in the PACTOR mode.

Directly under this you will see either "Recv" or "Xmit" to

indicate whether you are in the "Receive" or "Transmit" mode.

100 indicates that the current PACTOR speed is 100 baud.  This

status indicator will read either 100 or 200 depending on the

speed of the PACTOR link.

Huff indicates that Huffman compression is being used either on

the text being received or on your transmitted text.  The use

of Huffman Compression on transmit is controlled by the PTHUFF

command found on the PACTOR parameter setup screen.

18-2

Mdm indicates whether or not PACTOR MailDrop activity will be

monitored.  The default is OFF, which means that normal

monitoring is determined by the monitoring mode commands you

defined during the Setup process.  Press <F7> to change to ON.

Tmail: indicates the status of the PACTOR Mailbox (TMAIL) parameter.

In addition, both the current time and UTC time are shown in the

lower right-hand corner in military format.  UTC time is followed

by the letter Z.

There are also a number of function keys and other key combinations

that are active when you are operating in PACTOR mode:

ALT-M:Macro Keys lets you use any of the macros you've previously

defined.

CTRL-PgUp:Clear Buffer erases the contents of the scroll-back

buffer.

ALT-L:MiniLog takes you to the mini-log (it will be displayed at

the bottom of the screen).

PgUp:Scroll Back allows you to scroll through the data in the top

section.  You can press <F2> to save the information to disk.

You will be prompted for a file name.  The system will assume

you want the file saved in the current directory, so if you

want to save it to another location, type in the full path name.

Home:Log takes you to the QSO Log program.

<ESC> returns you to the Communications Menu.

PACTOR OPERATION

If you have never operated PACTOR before, it is a good idea to

listen to a few QSOs in the PTLIST(en) mode before initiating a

contact yourself.

You must press the the <F10> key to enter the PACTOR Listen mode to

monitor other stations.  The on-screen status display will show

that the data controller is now in PTLIST mode and is attempting to

phase with the received signal as shown below.

�����ALT-H:Help���ALT-M:Macro Keys���CTRL-Pg-Up:Clear Buffer����Ͷ

� PACTOR      100                        Mdm:OFF TMail:OFF xx:xx �

� Recv                     PTLIST  Phasing   PT200:ON      xx:xxZ�

�___ALT-L:MiniLog____PgUp:Scroll Back____Home:Log____xx/xx/xx____�

����������������������������������������������������������������ͼ

The best place to monitor other PACTOR stations is between 14.070

and 14.090 MHz on the 20-meter amateur band.  Listen for the 1-

second "cricket-sound" (i.e., "chirp chirp") of ARQ PACTOR signals.

18-3

To send a PACTOR CQ from the PACTOR Operating Screen, perform the

following steps:

1.   Make sure that MYPTCAL is defined in the Setup Screen.  To

check, press <ALT><K> and the PACTOR Setup Screen will be

displayed.  If there is nothing in the MYPTCAL field, then

move to that field and type it in now.  Type in your call sign.

Actually you may type up to 8 alphanumeric characters in this

field.  Press <ESC> to return to the PACTOR Operating Screen.

Don't forget to save the parameters in the Files Menu or you'll

have to enter the MYPTCAL callsign each time the program is

run.

2.   Press <F3>.  This will put the data controller into the

"PTSEND" mode.  You can verify this by checking the on-screen

status display or the front panel of your data controller.

3.   Type as many CQs as you want, followed by de followed by your

call sign.  For example:

CQ CQ CQ CQ de yourcall yourcall yourcall yourcall K K K

It doesn't matter how many times you type CQ.

Be sure to type "de" before you type your call sign.

Type your call sign at least once.

After the last CQ line, type KKK  (for "Over").

You can use a keyboard macro to accomplish all of this.

Refer to Section 13 of the PC-PAKRATT II Operating Manual,

CREATING AND USING MACROS.

4.   Press <CTRL><D> to end the transmission and transfer the data

controller to "Receive" mode.  You can even build that into

your macro.

5.   After you have called CQ, listen carefully to the receiver for

the sound of another station calling you.  PACTOR stations

normally respond to a CQ in ARQ PACTOR (A 1-second on-off

chirping sound will be heard) which will cause your

transmitter to automatically turn on and off about once per

second.  If you need to fine-tune the calling station, be sure

to use the transceiver's RIT to avoid changing your transmit

frequency.  When you see your on-screen status change from

Recv to Xmit, you'll know that the other station has turned

over control of the conversation to you.

6.   Type your part of the conversation in the bottom portion of

the screen.  As you type, you will see what you are typing

echoed in the "Receive" portion of the screen at the same time

the other station receives the data.

18-4

7.   When you are finished, press <END> to transmit the <CTRL-Z>

end-of-transmission symbol and let the other station know you

are returning control of the conversation to him.

PACTOR operation is a two-person, one-way connection.  When you are

transmitting, the other party may not transmit.  When you are

through typing your message, the <CTRL-Z> will let the other party

know that you are returning control to him.  Remember, when you are

transmitting, the on-screen status indicator and the front panel of

the data controller will indicate you are the sending station.

When you are receiving, the Recv and other on-screen indicators

will be on depending on exactly what is occurring on the Link.

PACTOR communications are conducted in Unprotocol and Connected

(ARQ) modes.  When you press <F3> to start PACTOR operation, you

send your CQs in Unprotocol PACTOR.  This is similar to FEC AMTOR

operation.  When you switch to "Receive" you are ready to start a

connected mode contact.  This is similar to ARQ AMTOR operation.

To link with another PACTOR operator whose CQ signal you pick up,

perform the following steps:

1.   When you scan the airwaves and find a PACTOR station with whom

you'd like to communicate, be sure to note the callsign.

2.   Press <F4>.

3.   Type the callsign that you noted and press <ENTER>.

4.   Check your status in the status portion of the screen.  When you

achieve a successful link, you will see the "Connected" message.

5.   Repeat steps 5-7 from the previous description for the

duration of the conversation.

To terminate a PACTOR link after saying "73", press <CTRL><D> or

<F8>.  This will completely break the link so be sure the other

party is expecting it.

PACTOR HELP SCREEN

When you are operating in the PACTOR mode, you can press <ALT><H>

to see the PACTOR Help screen.  Use the <PAGE UP> and <PAGE DOWN>

keys to view additional information.  It contains a list of the

function keys and function key combinations and a brief description

of what they do.  Here's the same information with a little more

explanation:

<F1> Open/Close Log File.  When you press this function key, you'll

will begin saving the contents of the Receive screen as it is

displayed and continue saving it until you press <F1> again.

You will be prompted for a CAPTURE file name.  To save data

that you have already captured in the QSO buffer, press the

<PAGE UP> key and do this from the scroll-back buffer.

18-5

<Shift F1> Start/Stop File Upload.  This function key combination

allows you to upload any of five message buffers or any text

file that you specify.

<CTRL F1> View a File.  This function key combination lets you look

at message or note files or any text file that you specify.

<ALT F1> Hold/Release Log File.  When you press these keys, you

pause (or resume) saving text to a log file.

<F2> MDCHECK/Mail Menu.  This function key moves you to the Mail

mode and displays the Mail menu.

<Shift F2> Open/Close Printer.  Starts and stops the printing of Rx Window

data.  When you are printing, you will see PRN in the status area.

<ALT F2> Hold/Release Printer.  This combination allows you to

temporarily prevent the printer from printing.  You will see

HOLD in the status area.

<F3> Unproto PACTOR Call.  When you want to call "CQ" in PACTOR or

want to communicate with another PACTOR station located very

close to you, press <F3> and begin your transmission.

<F4> PACTOR Connect.  When you press this key, you will be prompted

for the callsign of the station whose CQ you want to acknowledge.

<ALT-F6> TCLEAR.  This clears the transmit buffer of the data controller.

<F7> Toggle MDMON.  This turns the MDM parameter on and off.  The

status will be displayed in the lower portion of the screen.

<Shift F7> Toggle TMAIL.  This turns the PACTOR mailbox off and on.

<ALT F7> FREE.  When you press this key combination you will see a

message in the Receive portion of the screen that tells you

how many bytes are free in MailDrop.

<F8> PACTOR Standby.  This places the data controller in the PACTOR

standby mode.  The word "Standby" will appear in the bottom

line of the status area.

<F9> ACHG.  This forces a changeover in a connected ARQ PACTOR

contact.  It grabs the link so you can become the transmitting

station.  Don't do this unless you really need to.

<F10> PTLIST.  This key places the system in PACTOR-L(isten) mode.

<ALT-B> Toggle Break-In Buffer.  This allows you to insert some

text before anything you might have typed into the transmit

buffer is sent.

<ALT-C> Clear Rx Window.  This clears the Receive screen of all

data.

18-6

<ALT-D> Shell to DOS.  This will take you to the DOS prompt.  When

you are through doing whatever it was that you needed to do at

the DOS prompt, just type EXIT and press <ENTER> to return to

the PACTOR screen.  Do not stay shelled to DOS for extended

periods.  The data controller may lose data.

<ALT-E> Call Editor.  This key combination brings up a menu that allows

you to edit any of 5 messages, 2 notes or the text file of your

choice.  If you do not have enough free memory to load the editor,

you will see an error message informing you of the deficiency.

<ALT-L> Call MiniLog.  This will bring up the mini-QSO log at the

bottom of the screen.

<ALT-M> Define Keyboard Macros.  This lets you create macros for

use in PACTOR or any other mode of operation.

(Refer to Section 13 of the PC-PAKRATT II Operating Manual,

CREATING AND USING MACROS.)

<ALT-K> Parameter Setup Screen.  This will take you to the PACTOR

Setup screen.

<ALT-S> SETUP Screens.  This takes you to the Configuration Menu so

you can choose the Setup screen you wish to look at or modify.

<ALT-T> Clear Tx Window.  This clears the Send window of all data.

<ALT-Z> Change Mode.  This allows you to switch to another

operating mode (e.g., Packet, Baudot)

<ALT-1 through 0> Execute Keyboard Macro.  This allows you to accomplish

in one (or two or a few) keystrokes what might otherwise have

required many.  It invokes the macros you defined using <ALT><M>.

Refer to Section 13 of the PC-PAKRATT II Operating Manual

(CREATING AND USING MACROS) for more information.

<END> ^Z.  This terminates the transmission but not the link.  It

causes the <CTRL-Z> to be sent which changes the direction of

the PACTOR link and turns control of the conversation over to

him.

<HOME> Call QSO Log Facility. This takes you to the full QSO

logging program.

<PgUp> Scroll Back Buffer.  This allows you to scroll through the

Receive data that has been stored in the Scroll Back Buffer.

<CTRL-PgUp> Clear Scroll Back Buffer.  This allows you to erase the

Scroll Back Buffer.

<ESC> Return to Communications Menu.  This exits PACTOR mode and

returns to the Communications Menu.  If you are looking at the

PACTOR Help Screen when you press <ESC>, you will return to

the PACTOR screen.

18-7

#######################################################################################